Opis techniczny przedmiotu zamówienia

Advanced system of controllable motorized X-Y-Z stage/arm units with the necessary integrated safety and monitoring features meeting the following key requirements:

- Travel range: X ≥ 400 mm, Y ≥ 400 mm, Z ≥ 200 mm

- Positioning accuracy: ±40 µm

- Repeatability: ±20 µm

- Maximum travel speed: ≥120 mm/s

- Payload (Z-axis): ≥5 kg

- Closed-loop control with encoders

- Rigid aluminum frame with capability for mounting additional equipment (cameras, sensors, probes, etc.) without frame modification

- Safety enclosure with:

  - air filtration (HEPA H13 or better)

  - controlled relative humidity (down to around 25%)

  - door interlock and emergency stop

  - environmental sensors: temperature, relative humidity, atmospheric pressure, particulate matter PM1 & PM2.5

- Software: multi-axis control, monitoring & data logging, remote access

- On-site installation and training,

- Warranty: minimum 12 months with on-site warranty service

In your quotation, please specify:

- Technical compliance with the above requirements

- Price (net and gross)

- Delivery terms and lead time

- Installation, training, and after-sales service conditions

- Warranty period and coverage

- Available optional modules or upgrades